# Project Plan

## Introduction & Motivation:

Hotwire cutting is an automated manufacturing process where a heated wire usually made of nichrome is used to cut simple profiles out of a suitable parent material(Usually and expanded or extruded polymer product). Cutting is accomplished by heating the material to the point that the material thermally decomposes into gaseous products which naturally evacuate the cutting region, it follows that the size of the cut is proportional to the heat delivered at a point along the cut, a greater heat flux or a slower cut will increase the total thermal energy delivered to region of the parent material and so will result in a greater kerf (width of cut). Additional contributing factors result from high curvature geometries and thin geometries. The result is the when computing the tool path assuming a fixed kerf width results in distorted geometries.

By modelling the shape to be manufactured as a ruled surface defined by sets of parametrically defined b-splines the optimisation problems dimensionality can be reduced. Validation will be accomplished by comparing the measured geometry of test articles with the ground truth dimensions. In addition to testing the final product various other methods will be considered to show the relative optimisation performance.

Hotwire cutters are frequently used as a method for rapid manufacture and prototyping of Unmanned Ariel Vehicles (UAV’s) (Adawy, 2023) and other foam products, current software cannot correct for deviations in the manufactured shape from the designed shape. In many industrial uses there deviation are acceptable due to the scale or simple geometry of the manufactured product. Similarly for much research scales are large and so the dynamics can be safely ignored, further when reviewing past literature little is presented of how researchers have validated that the manufactured geometries match the designed geometries. In some research error data is presented, Duenser(2023) presented a quantitative examination of cutting errors, there use of a hot blade and low cutting speeds of 2mm/s allowed for surface geometries within 2% of the designed geometry, although this research is not directly transferable it shows that low errors are possible and presents useful metrics when approaching this problem and a potential target.

## Aim:

TO develop a method to optimise hotwire trajectory to minimise deviation from the designed shape.

## Objectives:

OB1. Understand sources of shape errors, quantify and characterise them.

OB2. Design a model to predict errors in shape.

OB3. Research, develop, and implement a method for optimising wire trajectory.

OB4. Quantify differences in performance between the 3 correction methods, How well each method reduces error, how the errors effects foil performance.

OB5. Quantify the performance of the new algorithm.

## EDIA considerations

Given the social impact of this topic is unlikely to be significant as a result of findings associated with its results and execution consideration focus primarily on accessibility.

Consideration for font spacings, and font style, will be made for those with reading difficulties. A colour palette that is more accessible for those with particular colour blindnesses will be chosen, if possible different version of the report with different colour schemes will be available to cater to those who have Tritanomaly as this colour scheme is mutually exclusive to those with Deuteranomaly and Protanomaly. Digital tools are available that can be used to simulate different visual difficulties, these will be used to inform the final colour scheme and style of presentation.

# Project Timeline and work packages

Below is an initial project timeline, much of the timeline in second semester is absent due to the uncertainty inherent any timeline.

| Work Package 0: | Participants: |
| --- | --- |
| Project management | Author |
| Objective: | Start: 28th September End: Ongoing |
| Inputs: |  |
| Supervisor input Previous experience |  |
| Tasks: |  |
| Develop Project timeline. Consider required resources, support and upskilling necessary. |  |
| Outputs: |  |
| Initial project timeline, initial schedule of meetings. |  |

| Work Package 1: | Participants: |
| --- | --- |
| Testing | Author |
| Objective: OB1 and OB4 | Start: 28th September End: Mid Semester 2 |
| Inputs: |  |
| Previous experience, Supervisor input |  |
| Tasks: |  |
| Design Standard Test article, - what will best demonstrate improvement? - what will be reasonable performance? - is this a geometry that is within the capability of this manufacturing method? Manufacture test article with fixed kerf width and old adaptive kerf width method. Get measurements of surface shape from 3D print lab 3d scanner or CMM machine. Compare measurement to ground truth. Design wind tunnel mounts. Test manufactured foils in subsonic project tunnel, gather data on wing performance. Compare measured results with theoretical and ground truth results. |  |
| Outputs: |  |
| Geometry and wind tunnel data |  |

| Work Package 2: | Participants: |
| --- | --- |
| Deliverables | Author |
| Objective: All of objectives | Dates: see below |
| Inputs: |  |
| Supervisor input |  |
| Tasks: |  |
| Skill development and project plan (23rd September - 21st October) - A document detailing the project outline including objectives and central aim Abstract (28th October – 23rd November) - Details to be decided but a paper abstract fit for submission to a relevant conference Seminar Presentation (27th January – 31st march) - A technical presentation to a group of professionals Non-technical summary(27th January – 5th may) - A non-technical summary of work performed |  |
| Outputs: |  |
| The requested set of deliverables |  |

| Work Package 3: | Participants: |
| --- | --- |
| Bureaucracy | Author |
| Objective: All objectives | Start: Ongoing |
| Inputs: |  |
| None |  |
| Tasks: |  |
| - Speak with Wind Tunnel Technicians to get wind tunnel time. - Speak with the 3D print lab confirm access to the 3D scanner - Speak with Robert Hyneman request access to the CMM machine |  |
| Outputs: |  |
| Date and time for tunnel testing Process to book use of 3d Scanner |  |

| Work Package 4: | Participants: |
| --- | --- |
| Research and Development | Author |
| Objective: OB2, OB3, and OB4 | Start: September End: |
| Inputs: |  |
| Supervisor input Problem areas identified in skill reflection |  |
| Tasks: |  |
| Literature review - Review relevant papers. - What is the state of the art, have others attempted similar in the past. - What methods would be suitable for this project. - How can the loss function be defined, how will it drive the optimisation. Play with chosen method - Gain familiarity with the chosen method. - How can the method be efficiently implemented. - How will potential limits on dimensionality be addressed. Implementation - Implement the chosen method integrating with old UI. - Analise the performance of the new method. - How fast does the program execute. - How does its performance depend on the input condition. - Are there any anachronisms to the method, potential flaws? |  |
| Outputs: |  |
| A chosen method for approaching the project. An understanding of a method of implementation of the method. A subroutine that can perform optimisation of the chosen airfoil. Quantitative information about how the subroutine performed and scaled. |  |

| Work Package 5: | Participants: |
| --- | --- |
| Education | Author |
| Objective: OB2 and OB3 | Start: 28th September End: Ongoing |
| Inputs: |  |
| Supervisor input Previous experience |  |
| Tasks: |  |
| Back propagation - Gain a theoretical understanding. - Gain a practical understanding of its implementation. ADAMS - Gain a theoretical understanding. - Gain a practical understanding of its implementation. B-Splines - Gain a theoretical understanding. - Gain a practical understanding of its implementation. |  |
| Outputs: |  |
| A working understanding of mathematical tools relevant to the project. |  |

## Reflection

There are several potential conflicts that may arise, a central part of this project is the comparative analysis, if the Wind Tunnel or 3d scanners become unavailable a central aim of the project would be jeopardised. Excluding total mechanical failure this can be avoided with early booking of the wind tunnel with a redundant plan if that is not possible, one of which would be using pressure tapping instead of direct force measurement, or importing the measured geometry into a CFD package and using a simulation.

A key piece of feedback provided was that the project objectives were unclear, have now had the opportunity to meet with my advisor I have been able to gain a more clear understanding of the project area and approaches to this problem. Since the meeting the project objectives have been iterated.

# Skill Development Plan:

## Reflection:

I have significant experience in python and in use of hotwire machines for manufacturing, as part of my dissertation I gained experience with computer optimisation, this should help in keeping the development time line of a reasonable length.

I already have a large package of supporting code. Integrating new code into old code is notoriously challenging, it is possible that implementation may take far longer than expected and cause a significant over run. Having looked at the code, it is lacking in some ways and part will have to be refactored, this will need to be accounted for in the time estimates. This task is well within my abilities and should not take very long.

## Skill development

Time has been allotted and specific work packages made that focus on working with the variety of mathematical tools needed for this project, hopefully with will reduce the number of false starts when developing the software. This was a problem I experienced repeatedly during my individual project and caused huge delays that resulted in the software only being finished a week or so before the final deadline. Given the dependency of testing on completion of the code this isn’t acceptable. The steps taken should deal much of the problems highlighted in the reflection.

My experience with optimisation has focused on problems that are not similar to this problem, my experience lies with single DOF continuous systems, or n-DOF systems with binary design variables. This problem is described by a n-DOF system with continuous design variables . Although a very well-studied space I do not yet understand it well enough to make best use of prior work.

A persistent problem with my academic work is my weakness In academic writing, although not much can be done to remedy the root cause it can be offset by submitting more drafts for feedback, extending the writing timeline will also help me catch more mistakes in writing and to better communicate concepts.

# Relevant Citations

Duenser, S., Poranne, R., Thomaszewski, B., and Coros, S., 2020. Robocut: Hot-wire cutting with robot-controlled flexible rods. ACM Transactions on Graphics (TOG), 39(4), pp.98-1. Available at: https://crl.ethz.ch/papers/hotwirecutter.pdf (Accessed 25/10/2024)

El Adawy, M., Abdelhalim, E.H., Mahmoud, M., Mohamed, I.H., Othman, M.M., ElGamal, G.S., and ElShabasy, Y.H., 2023. Design and fabrication of a fixed-wing Unmanned Aerial Vehicle (UAV). Ain Shams Engineering Journal, 14(9), p.102094. Available at: https://www.sciencedirect.com/science/article/pii/S2090447922004051 (Accessed:25/10/2024)

Figliolini, G., Rea, P., and Cocomello, C., 2018. Mechatronic design and prototype of a 4-DOFs hot-wire CNC cutting machine. In Advances in Service and Industrial Robotics: Proceedings of the 26th International Conference on Robotics in Alpe-Adria-Danube Region, RAAD 2017 (pp. 591-598). Springer International Publishing. Available at: https://link.springer.com/chapter/10.1007/978-3-319-61276-8_62 (Accessed: 25/10/2024)

Karmakar, N., and Subbiah, S., 2018. Investigating Bowing of Hot Wire during cutting of EPS. Procedia Manufacturing, 26, pp.671-680. Available at: https://www.researchgate.net/publication/326822350_Investigating_Bowing_of_Hot_Wire_during_cutting_of_EPS (Accessed: 25/10/2024)

Mollica, E., and Timmoneri, A., 2021. CFD analysis of the low Reynolds S1223 airfoil. International Journal of Engineering, Science and Technology, 13(4), pp.46-49. (Accessed: 25/10/24)

Wang, C., 2019. MAEG5030: Geometric Computing for Design and Manufacturing. Available at: https://mewangcl.github.io/Courses/CUHKGeoCompDM.htm (Accessed: 25/10/24)