# Project Plan

## Introduction & Motivation:

Hotwire cutting is an automated manufacturing process where a heated wire usually made of nichrome is used to cut simple profiles out of a suitable parent material(Usually and expanded or extruded polymer product). Cutting is accomplished by heating the material to the point that the material thermally decomposes into gaseous products which naturally evacuate the cutting region, it follows that the size of the cut is proportional to the heat delivered at a point along the cut, a greater heat flux or a slower cut will increase the total thermal energy delivered to region of the cut surface and so will result in a greater kerf(width of cut). Additional contributing factors result from high curvature geometries and thin geometries. The result is the when computing the tool path assuming a fixed kerf width results in distorted geometries.

By modelling the shape to be manufactured as a ruled surface defined by sets of parametrically defined b-splines the optimisation problems dimensionality can be reduced. Validation will be accomplished by comparing the measured geometry of test articles with the ground truth dimensions. In addition to testing the final product various other methods will be considered to show the relative optimisation performance.

## Aim:

Develop a method to optimise hotwire trajectory to minimise surface shape errors.

## Objectives:

OB1. Undertake a testing campaign to quantify magnitude and types of errors for standard tool paths, and for the existing method for correcting these errors

OB2. Research, Develop and implement the a new approach.

OB3. Compare manufacturing quality, computational requirements, and perform trade off analysis.

OB4. Perform Wind Tunnel testing to compare aerodynamic performance.

## EDIA considerations

Given the social impact of this topic is unlikely to be significant as a result of findings associated with its results and execution consideration focus primarily on accessibility.

Consideration for font spacings, font, will be made for those with reading difficulties. A colour palette that is more accessible for those with particular colour blindnesses will be chosen, if possible different version of the report with different colour schemes will be available to cater to those who have Tritanomaly as this colour scheme is mutually exclusive to those with Deuteranomaly and Protanomaly.

Tool like https://www.color-blindness.com/coblis-color-blindness-simulator/ will be used to check accessibility

# Project Timeline

Below is an initial project timeline, much of the timeline in second semester is absent due to the uncertainty inherent any timeline.

Included for completeness is the current set of Project work packages, boxes coloured solid green have been completed.

Work packages do not align 1:1 with the Gantt chart to include all the work packages would clutter the Gantt chart and reduce its usefulness.

| Work Package 0: | Participants: |
| --- | --- |
| Project management | Author |
| Objective: | Start: 28th September End: Ongoing |
| Inputs: |  |
| Supervisor input Previous experience |  |
| Tasks: |  |
| Develop Project timeline Consider required resources, support and upskilling necessary |  |
| Outputs: |  |
| Initial project timeline, initial schedule of meetings |  |

| Work Package 1: | Participants: |
| --- | --- |
| Testing | Author |
| Objective: OB.1 | Start: September End: |
| Inputs: |  |
| Previous experience, Supervisor input |  |
| Tasks: |  |
| Design Standard Test article, - what will best demonstrate improvement - what is reasonable to be expected - is this a geometry that is within the capability of this manufacturing method Manufacture test article with fixed kerf width and old adaptive kerf width method Get measurements of surface shape from 3D print lab 3d scanner or CMM machine Compare measurement to ground truth Design wind tunnel mounts Test manufactured foils in subsonic project tunnel, gather data on wing performance Compare measured results with theoretical and ground truth results |  |
| Outputs |  |
| Geometry and wind tunnel data |  |

| Work Package 2: | Participants: |
| --- | --- |
| Deliverables | Author |
| Objective: | Start: End: see below |
| Inputs: |  |
| Supervisor input |  |
| Tasks: |  |
| Skill development and project plan (23rd September - 21st October) - A document detailing the project outline including objectives and central aim Abstract (28th October – 23rd November) - Details to be decided but a paper abstract fit for submission to a relevant conference Seminar Presentation (27th January – 31st march) - A technical presentation to a group of professionals Non-technical summary(27th January – 5th may) - A non-technical summary of work performed |  |
| Outputs |  |
| The requested set of deliverables |  |

| Work Package 3: | Participants: |
| --- | --- |
| Bureaucracy | Author |
| Objective: All objectives | Start: Ongoing |
| Inputs: |  |
| None |  |
| Tasks: |  |
| Speak with ozgun and Andrew to get wind tunnel time. Speak with the 3D print lab confirm access to the 3D scanner Speak with Robert Hyneman request access to the CMM machine |  |
| Outputs: |  |
| Date and time for tunnel testing Process to book use of 3d Scanner |  |

| Work Package 4: | Participants: |
| --- | --- |
| Research and Development | Author |
| Objective: OB2, OB3 | Start: September End: |
| Inputs: |  |
| Supervisor input Problem areas identified in skill reflection |  |
| Tasks: |  |
| Literature review - Review relevant papers. - What is the state of the art, have others attempted similar in the past. - What methods would be suitable for this project. - How can the loss function be defined, how will it drive the optimisation Play with chosen method - Gain familiarity with the chosen method - How can the method be efficiently implemented - How will potential limits on dimensionality be addressed Implementation - Implement the chosen method integrating with old UI - Analise the performance of the new method - How fast does the program execute - How does its performance depend on the input condition - Are there any anachronisms to the method, potential flaws? |  |
| Outputs |  |
| A chosen method for approaching the project An understanding of a method of implementation of the method A subroutine that can perform optimisation of the chosen airfoil Quantitative information about how the subroutine performed and scaled. |  |

# Reflection and Skill Development Plan:

## Reflection:

I have significant experience in python and in use of hotwire machines for manufacturing, as part of my dissertation I gained experience with computer optimisation, this should help in keeping the development time line of a reasonable length.

I already have a large package of supporting code. Integrating new code into old code is notoriously challenging, it is possible that implementation may take far longer than expected and cause a significant over run. Having looked at the code, it is lacking in some ways and part will have to be refactored, this will need to be accounted for in the time estimates. This task is well within my abilities and should not take very long.

my experience with optimisation has focused on problems that are not similar to this problem, my experience lies with single DOF continuous systems, or nDOF systems with binary design variables. This problem is described by a nDOF system with continuous design variables . Although a very well-studied space I do not yet understand it well enough to make best use of prior work.

A persistent problem with my academic work is my weakness In academic writing, although not much can be done to remedy the root cause it can be offset by submitting more drafts for feedback, extending the writing timeline will also help me catch more mistakes in writing and to better communicate concepts.

## Skill development

Time has been allotted and specific work packages made that focus on working with the variety of mathematical tools needed for this project, hopefully with will reduce the number of false starts when developing the software. This was a problem I experienced repeatedly during my individual project and caused huge delays that resulted in the software only being finished a week or so before the final deadline. Given the dependency of testing on completion of the code this isn’t acceptable. The steps taken should deal much of the problems highlighted in the reflection.